Conservation Discussions

By Bryon Kirwan, State Economist, Illinois

Defining and Understanding Land Measurements

When we are working in the field, we often measure by common terms such as feet, acres, miles, etc.  However you may come across a wide variety of valid (although less used today) types of measures.  Knowing what these measures mean, how to interpret them, and how to apply them is important.

Some of these measures pre-date the settling of America.  Others were used by early surveyors as they measured land, and as such, are a part of the legal record of a parcel. There were differences in some of these measures in different parts of the Unites States.  Other uses of these measures can be found in price estimates.  For example, some fence contractors would make their installation estimates on a “Dollars per Rod” basis.

Land Measures

Regarding land areas, one of the oldest measures is “Metes and Bounds”.   It is described as: (metes and bounds) n. a surveyor's description of a parcel of real property, using carefully measured distances, angles, and directions, which results in what is called a "legal description" of the land, as distinguished from merely a street address or parcel number. Such a metes and bounds description is required to be recorded in official county record on a subdivision map and in the deeds when the boundaries of a parcel or lot are first drawn.  Copyright © 1981-2005 by Gerald N. Hill and Kathleen T. Hill. All Right reserved.

Metes—a boundary or boundary stone; perimeter descriptions with a direction of travel described by a series of measurements

Bounds—perimeter descriptions without a direction of travel

The other measure of land areas (and the one that is dominant in the central and western US) is the Rectangular System of Public Land Survey.  This measure provided for the division of land into townships, sections, and quarter sections.  This system utilizes township lines and range lines as a part of the description.  An example would be: the E ½ of the NE ¼ Section 31, T6N, R6W.  This would be a description of 80 acres.  This system followed the Metes and Bounds surveys, and was established in 1785.

Some other units of measure you could come across include:

Rods-- Unit of length and area. Also known as a perch or pole. As a unit of length, equal to 16.5 feet.

Chains—a measure typically understood to be Gunter's chain, but could be of another type in different areas.  A chain was made of 100 links of a known length.  Gunter's chain is 4 rods long, otherwise noted as 66 feet long

Links—links are the individual measure of the chain.  As a chain has 100 links, Gunter's chain links would be 7.92 inches in each link.

Furlong—an area equal to 40 rods in length.  This is also expressed as 660 feet, 220 yards, or 1/8 of a mile. This term is still used commonly in horse racing.

Other descriptions of survey measures include: (source: Clinton County, Ohio GIS)

Acre - The (English) acre is a unit of area equal to 43,560 square feet, or 10 square chains, or 160 square poles. It derives from a plowing area that is 4 poles wide and a furlong (40 poles) long. A square mile is 640 acres. The Scottish acre is 1.27 English acres. The Irish acre is 1.6 English acres.

Arpent - Unit of length and area used in France, Louisiana, and Canada. As a unit of length, approximately 191.8 feet (180 old French 'pied', or foot). The (square) arpent is a unit of area, approximately .845 acres, or 36,802 square feet.

Chain - Unit of length usually understood to be Gunter's chain, but possibly variant by locale. See also Rathbone's chain. The name comes from the heavy metal chain of 100 links that was used by surveyors to measure property bounds.

Compass - One toise.

Engineer's Chain - A 100 foot chain containing 100 links of one foot apiece.

Furlong - Unit of length equal to 40 poles (220 yards). Its name derives from "furrow long", the length of a furrow that oxen can plow before they are rested and turned. See Gunter's chain.

Gunter's Chain - Unit of length equal to 66 feet, or 4 poles. Developed by English polymath Edmund Gunter early in the 1600's, the standard measuring chain revolutionized surveying. Gunter's chain was 22 yards long, one tenth of a furlong, a common unit of length in the old days. An area one chain wide by ten chains long was exactly an acre. In 1595 Queen Elizabeth I had the mile redefined from the old Roman value of 5000 feet to 5280 feet in order for it to be an even number of furlongs. A mile is 80 chains.

Hectare - Metric unit of area equal to 10,000 square meters, or 2.471 acres, or 107,639 square feet.

Hide - A very old English unit of area, a hide was of variable size depending on locale and the quality of the land. It was the amount of land to support a family, and ranged from 60 to 180 acres. After the Norman conquest in 1066 it became standardized at around 120 acres.

Hundred - An administrative area larger than a village and smaller than a county. In England it was 100 hides in size, and the term was used for early settlements in Virginia, Maryland, and Delaware.

Labor - The labor is a unit of area used in Mexico and Texas. In Texas it equals 177.14 acres (or 1 million square varas).

League (legua) - Unit of area used in the southwest U.S., equal to 25 labors, or 4428 acres (Texas), or 4439 acres (California). Also, a unit of length-- approximately three miles.

Link - Unit of length equal to 1/100 chain (7.92 inches).

Out - An 'out' was ten chains. When counting out long lines, the chain carriers would put a stake at the end of a chain, move the chain and put a stake at the end, and so on until they ran "out" of ten stakes.

Perch - See pole.

Point - A point of the compass. There are four cardinal points (North, South, East, West), and 28 others yielding 32 points of 11.25 degrees each. A survey line's direction could be described as a compass point, as in "NNE" (north northeast). To improve precision, the points would be further subdivided into halves or quarters as necessary, for example, "NE by North, one quarter point North". In some areas, "and by" meant one half point, as in "NE and by North".

Pole - Unit of length and area. Also known as a perch or rod. As a unit of length, equal to 16.5 feet. A mile is 320 poles. As a unit of area, equal to a square with sides one pole long. An acre is 160 square poles. It was common to see an area referred to as "87 acres, 112 poles", meaning 87 and 112/160 acres.

Pueblo - A Spanish grant of less than 1000 acres.

Rancho - A Spanish grant of more than 1000 acres.

Rathbone's Chain - A measuring chain two poles, or 33 feet, in length.

Rod - See pole

Rood - Unit of area usually equal to 1/4 acre.

Toise - Traditional French unit of length equal to 6 old French 'pieds' or feet, or 6.4 English feet.

Vara - Unit of length (the "Spanish yard") used in the U.S. southwest. The vara is used throughout the Spanish speaking world and has values around 33 inches, depending on locale. The legal value in Texas was set to 33 1/3 inches early in the 1900's.

Virgate - An old English unit of area, equal to one quarter of a hide. The amount of land needed to support a person.

Units of Measure and Conversion Factors

| One | Inches | Feet | Yards | Statute Miles | Nautical Miles | mm |
| --- | --- | --- | --- | --- | --- | --- |
| Inch | 1 | 0.0833 | 0.0277 |  |  | 25.4 |
| Foot | 12 | 1 | 0.333 |  |  | 304.8 |
| Yard | 36 | 3 | 1 | 0.00056 |  | 914.4 |
| Statute Mile | 63,360 | 5,280 | 1,760 | 1 | 0.8684 |  |
| Nautical Mile | 72,963 | 6,080 | 2,026 | 1.1516 | 1 |  |
| Millimeter | 0.0394 | 0.0033 | 0.0011 |  |  | 1 |
| Centimeter | 0.3937 | 0.0328 | 0.0109 |  |  | 10 |
| Decimeter | 3.937 | 0.328 | 0.1093 |  |  | 100 |
| Meter | 39.37 | 3.2808 | 1.0936 | 0.0006 | 0.0005 | 1,000 |
| Decameter | 393.7 | 32.81 | 10.94 | 0.0062 | 0.0054 | 10,000 |
| Hactometer | 3,937 | 328.1 | 109.4 | 0.0621 | 0.0539 | 100,000 |
| Kilometer | 39,370 | 3,281 | 1,094 | 0.6214 | 0.5396 | 1,000,000 |
| Myriameter | 393,700 | 32,808 | 10,936 | 6.2137 | 5.3959 | 10,000,000 |

Metric System of Linear Measure

| 1 | millimeter | = | 0.1 | centimeter | = | 0.0393 | inch |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | millimeter | = | 1.0 | centimeter | = | 0.3937 | inch |
| 10 | centimeters | = | 1.0 | decimeter | = | 3.937 | inches |
| 10 | decimeters | = | 1.0 | meter | = | 39.37 | inches |
| 10 | meters | = | 1.0 | decameter | = | 32.81 | feet |
| 10 | decameters | = | 1.0 | hectometer | = | 328.1 | feet |
| 10 | hectameters | = | 1.0 | kilometer | = | 0.62 | mile |
| 10 | kilometers | = | 1.0 | myriameter | = | 6.21 | miles |

Linear Measure/Long Measure				Square Measure

1 Mile		= 80 Chains				1 Square Mile		= 640 Acres

= 320 Rods

= 5280 Feet				1 Acre			= 10 Square Chains

= 160 Square Rods

1 Chain		= 4 Rods							= 43560 Square Feet

= 66 Feet

= 100 Links				1 Square Rod		= 30.25 Square Yards

= 272.25 Square Feet

1 Link		= .66 Feet

= 7.92 Inches				1 Square Foot		= 144 Square Inches

1 Pole		= 16.5 Feet

1 Furlong	= 220 Yards

= 1/8 Mile

* The Earth curves approximately 8 inches per mile.